Homework 12 Advanced Thermodynamics

Due Monday November 14, 2021

Nanocrystalline materials can have exceptional mechanical properties compared to conventional crystals. The yield strength can increase following the Hall-Petch relationship y = 0 + Kd-1/2. Nanocrystalline materials do not strain harden so they display perfect or super-plasticity. The modulus decreases by 30 to 50%. Grain growth or coarsening of nanocrystalline metals during processing has hindered their application. Saber M; Kotan H; Koch CC; Scattergood RO Thermodynamic stabilization of nanocrystalline binary alloys J. Appl. Phys. 113 063515 (2013) developed a predictive thermodynamic model for stabilization of nanocrystalline binary alloys using a regular solution model.

- Explain the general features of an ideal solution and a regular solution model. Derive the entropy and enthalpy expressions for these models using a statistical thermodynamics approach.

- Nanograin size can be explained by kinetic stabilization or by thermodynamic stabilization.  Explain the logic behind these two models. What is the advantage of a metastable state model?

- Weissmuller J Alloy Effects in Nanostructures Struc. Mat. B 261-272 (1993) in Figure 1 shows three possible scenarios for solid solutions involving grain boundaries. How does Saber et al. account for these three possibilities in their model?

- Introduction of a solute atom in a lattice leads to lattice strain. This strain could be relieved if the solute were located at a grain boundary. In addition to this elastic component, there is a chemical advantage to locating the solute at a grain boundary. Explain the origin of the chemical advantage and how it is incorporated in Saber’s model.

- Derive Saber’s equation 12.

- The coordination number, z, doesn’t seem to be constant between the interface and the grains in Figure 1 of Saber. Explain how this is accounted for in Saber’s model.

- Weissmuller (1993) shows a plot of G versus D (Figure 3) explain why minimization of Saber’s equations 29 a to c are necessary to find an “equilibrium” state, rather than just minimization with respect to the intergranular volume fraction, fig, as shown by Weissmuller. Is this really an equilibrium state?

- In Figure 5 of Saber, why does grain size increase with temperature, and why does it decrease with increasing ?