Chemical Engineering Thermodynamics

Quiz 2       January 21, 2021

The Diesel Cycle is a model engine to study unmixed fuel/air mixture engines typical for trucks. Diesel engines operate at a much higher compression ratio, about V1/V2 = 20, compared to a gas engine following the premixed (carburated/fuel injected) Otto Cycle.  Consider one cylinder of a four-stroke diesel engine with each cylinder having a VTDC = V1 = 583 cm3 (6 cylinders are 3.5L).  The stages are:

0-1 Intake		Isobaric

1-2 Compression	Adiabatic

2-3 Ignition		Isobaric

3-4 Power stroke	Adiabatic

4-1 Blowdown		Isochoric

1-0 Exhaust		Isobaric

0

Four Stroke Otto Cycle (not used)			 Four Stroke Diesel Cycle (Used Here)
(Both figures from Wikipedia)

https://web.mit.edu/16.unified/www/FALL/thermodynamics/notes/node26.html

Consider that the material in the cylinder is 2 mole percent isooctane in air with Cp = 3.89R for the mixture with a combustion enthalpy for pure octane of 5,470 kJ/mole (109 kJ/mole for the mixture).

Assume an ideal gas throughout the calculations.

Ignore the increase in number of moles with combustion.

- Solve for Pf and Tf for the 1-2 stroke.

(Fill in your answers in the table for each calculation.  The table should be filled out by the end of the quiz.  Include a sheet showing your work.)

- Solve for Vf in the 2-3 stroke.

- Solve for Pf and Tf for the 3-4 stroke.

- Calculate WEC, Q, the internal energy changes, U, and enthalpy changes, H, using CV and CP for all of the strokes that are not greyed out in the table. Keep in mind that R is in units of Joules not kJ. A Joule is equal to MPa cm3.

- Calculate the efficiency of this engine (net work/enthalpy input) if the fuel/air mixture had a combustion enthalpy of 5,470 kJ/mole * 0.02 = 109 kJ/mole of the mixed gas in the engine (accounting for 2% isooctane in air).  Keep in mind that R is in units of Joules not kJ.  A Joule is equal to MPa cm3.

You can use the attached excel sheet for your answers and calculations.  Make sure you write out your calculations on a separate sheet of paper so that I can follow your work.  Remember to use 3 significant digits and put units on every number you write down or put in the excel sheet (where possible).

1 atmosphere is 14.7 psi, 1.01 bar, 0.101 MPa, 760 mmHg, 29.9 inHg

Qrev = U for isochoric (constant volume)     			4.17

dU = Cv dT  for isochoric (constant volume)

Cp = Cv + R (exact for ideal gas)

H = U + (PV) = U + R(T) (exact for ideal gas)

ANSWERS:   Chemical Engineering Thermodynamics

Quiz 2

January 21, 2021