Chemical Engineering Thermodynamics

Quiz 1

January 16, 2020

Consider a simplified steam turbine/condenser/boiler/superheater for production of electricity shown in the schematic below.  Using the attached steam table, fill in the table below the diagram to answer the questions.

- Use the steam tables to determine the shaft work, Ws, for the turbine in kW (which is kJ/s).  Under and adiabatic assumption (no heat loss) the shaft work equals the difference in enthalpy, H, between the exiting and entering streams at 100% efficiency.  (The flow rate is 1200 kg/h for all streams.)

- Calculate the combined heat needed for the boiler and superheater (boiler converts from liquid to vapor and superheater further heats the steam) in kW (which is kJ/s).  (This is the enthalpy (H) difference between the streams.)

- Take the ratio of the shaft work recovered from the steam turbine to the heat needed for the boiler to get an idea of how efficient this system is with a turbine at 100% efficiency.  Why is this efficiency less than 100%?

- The maximum possible efficiency for a heat engine can be shown to be (TH – TC)/TH.  Compare this best possible efficiency to your efficiency.  (Carefully choose the units of temperature.)

- On the log-log P vs V plot, below, approximately show the points 1, 2 and 3.  Explain why the lever rule will not work on this plot.

Water Molecular Weight 18.0 g/mol

1 m3 = 106 cm3

Watt = J/s

Turn this sheet in with your answer

- Shaft Work =

- Combined Heat =

- Efficiency =                                        (Give comments/calculations on separate sheet)

- Ideal Efficiency =                               (Give comments/calculations on separate sheet)

ANSWERS:  Chemical Engineering Thermodynamics

Quiz 1          January 16, 2020

- Find H2-H1

Ws = (2580-3100)kJ/kg (1200 kg/h) (1/(3600 s/h)) = -173 kW

- Find H1-H3

Q = (3100-763)kJ/kg (1200 kg/h) (1/(3600 s/h)) = 779 kW

-  = 173 kW/779 kW = 0.222 or 22.2% efficiency.

There is loss at the condenser.  Also, there is a maximum efficiency associated with the necessary production of entropy limiting the process to about 27% efficiency, see part d.

- Carnot = (TH – TC)/TH = (350°C-180°C)/(350°C + 273°C) = 0.273

The Carnot efficiency is higher than the observed efficiency since this is a real process.

- The lever rule won’t work on a log-scale.